WSN based systems

The trends in the development of the IAQ monitoring system reveal that most of the researchers in the past few years have worked on WSN based designs with ZigBee as the most reliable communication protocol. The ATmega microcontroller manages the real-time data collection; however, Raspberry Pi is another common choice for setting up a sensor network in the target environment. WSN is an Ad Hoc Network, where sensor networks consume immense energy while transmitting data in multiple hops. The time taken by sensors to send a signal to the monitoring unit was observed to be considerably high. In such situations, researchers needed to work on battery power management to improve overall system performance. However, only a few researchers, such as Yu and Lin [36] were successful in implementing energy-saving and cost-saving monitoring systems using WSN architecture. Trends reveal that most of the WSN based IAQ monitoring systems use web servers as data access platforms; it demands additional efforts to generate real-time alerts on user smartphones to prevent hazardous conditions. Table 2 highlights the summary of WSN based IAQ monitoring systems.